# GPU-equipped compute machines within NTU IoT Sensing group

Lots of appreciation to Zhenyu, Wenjie, Duc, Dongfang, Jiale, Yuting, Zimo, Yimin, and Duc’s efforts in managing these compute resources

Current server coordinators: Dongfang, Zimo, Yuting, Yimin

# Server User Manual

## Connecting instructions:

The server is accessible via SSH  (command lines) and RDP (with graphic interface, use Microsoft Remote Desktop on both Mac and Windows, Remmina on Linux)

address: serverX.ntuiot.xyz

Note that the server is only accessible using NTU internal network, which means you can only connect to the above address via NTUSECURE wifi or ethernet cable in lab. In order to access the servers off campus, please consider using NTU VPN. Info about NTU VPN: https://ntuvpn.ntu.edu.sg/dana-na/auth/url_default/welcome.cgi

## [Important] Reserve in advance:

Please contact Rui for accessing the calendar. Please use the calendar to book your use of resources. Name the event by the resources (CPU, GPU) that you will use, specify your name and time durations.

Here are some examples:

“Zhenyu @ Server 1: CPU”

“Zhenyu @ Server 1: CPU + all GPUs”

“Wenjie @ Server 2: CPU + GPU0 + GPU1”

“Wenjie @ Server 3: 16 CPU cores”

The above booking is just for every user’s information. No restrictions are implemented. So, please try your best to proactively and honestly log your use of servers, to be considerate for others.

## [Important] Data storage:

DO NOT STORE BULKY DATA IN HOME FOLDER. Since user's home directories are on the same disk as the operating system, if the disk is full, the machine will go down. Hence, instead of putting everything in your home directory (~/ or /home/xxx/), please store your bulky files to /dataX/<username>, e.g., /data4/zhenyu/. Details of the data disks in each server can be found  in Shared resources below.

Also, please do not use 'pip install xxx --user', because it also occupies home folder storage. Use virtual environment instead.

## [Important] Sudo operations:

By default we do not grant sudo privilege to server users. If you intend to use any packege/software that is not pre-installed on this machine, please first check if it can be installed in your own directory. If it has to be installed with sudo privilege, please contact server admin for assistance. If sudo privilege is granted, please be VERY CAREFUL before you sudo anything. If any problem / malfunction is caused by the misuse of sudo, one should be responsible for recovering the server back to normality.

## [Important] Python Environments:

There are multiple versions of a python installed. However, please use virtual environments for your python project, which can prevent corrupting other users and avoid higher privileges. More info about virtualenv: https://virtualenv.pypa.io/en/latest/userguide/ or conda:https://docs.conda.io/en/latest/miniconda.html.

Besides, it is also recommended to manage your CUDA dependencies inside your virtual environment.

DO NOT touch the original python configuration of the OS. If you happen to encounter any environment problem, please try to fix it in your own virtual environment first.

Shared resources

- Tower Server 1 (Retired and returned to Mr. Toh, coordinator: Dongfang Guo <dongfang.guo@ntu.edu.sg>)

| Component | Description | Usage notes |
| --- | --- | --- |
| Domain name | server1.ntuiot.xyz |  |
| Motherboard | Asus X99-E WS Motherboard with integrated Intel® X99 chipset, integrated Realtek ALC1150 8-Channel HD Audio CODEC, integrated 2 x Intel Gigabit LAN Port, support 4 x PCIe x16 GPU Card |  |
| CPU | 6 core, 12 threads, 3.6GHz (up to 3.8GHz) Intel Core i7-6850K 3.60GHz 15MB Cache 6C/12T Processor up to 3.8GHz turbo + Intel Heatsink with Fan |  |
| RAM | 128GB - Corsair Vengeance LPX 32GB (2x16GB) DDR4 3200 C16 1.35V - Upgrade 1: Corsair Vengeance LPX 32GB (2x16GB) DDR4 3200 C16 1.35V - Upgrade 2: Corsair Vengeance LPX 32GB (4x16GB) DDR4 3200 C16 1.35V |  |
| SSD | 1.5TB - SSD0: 1 x 2.5” Samsung 960 PRO M.2 Series 512GB NVMe SSD Hard Drive - SSD1: Upgrade 1: SSD: 2.5” Samsung 860 Pro Series 1TB SSD (5 years limited warranty) | SSD0: Root (128 GB) and Home directories (250 GB shared) are sharing the same high-speed SSD. Don’t store any experiment data in home directory due to its limited capacity. SSD1 (1TB): mounted to /data4 Please store bulky hot data to /data4 |
| HDD | 12TB - HDD0: 1 x 3.5” Seagate IronWolf Pro 4TB NAS Drive 7200rpm (ST4000NE0025) - HDD1: Upgrade 1: Hard Disk Drive: 3.5” Seagate IronWolf 8TB 7200rpm 256MB Cache SATA Hard Disk Drive (3 years warranty) - HDD2: IronWolf 8TB 7200rpm 256MB Cache SATA Hard Disk Drive (3 years warranty) | HDD0 (4TB): mounted to /data HDD1 (8TB): mounted to /data2 HDD2 (8TB): mounted to /data3 (reserved for TDC project data, don’t use) Please store cold data to /data/<your name> and /data2/<your name> |
| PSU | Cooler MasterWatt Maker 1500W 80+ Titanium Power Supply |  |
| Case | Carbide Series® Air 540 High Airflow ATX Cube Case |  |
| GPU | Installed: - GPU0: PNY nVidia Quadro RTX 6000 24GB GDDR6 PCIe x16 GPU Card - GPU1: GTX Titan V 12GB (donated by NVIDIA) - GPU2: PNY nVidia Quadro RTX 6000 24GB GDDR6 PCIe x16 GPU Card Removed: - GV-N108TAORUS X-11GD 1 AORUS GeForce® GTX 1080 Ti Xtreme Edition 11G (installed on the Apollo machine. Apollo machine not operating now, kept by Toh.) - GV-N108TAORUS X-11GD 1 AORUS GeForce® GTX 1080 Ti Xtreme Edition 11G (installed on the LGSVLmachine) |  |
| Cost (S$, inc GST) |  | 4179.42 (server) + 2,047.98 (server upgrade1) + 759.70 (server upgrade2) + 3993.24 (3 GTX 1080 Ti) = 10,980.34 |

- Tower Server 2 (deployed in CNCL, coordinator: Ma Zimo <ZIMO001@e.ntu.edu.sg>)

| Component | Description | Usage notes |
| --- | --- | --- |
| Domain name | server2.ntuiot.xyz |  |
| Motherboard | Asus WS X299 SAGE Motherboard with integrated Intel X299 chipset, Realtek ALC S1220A 7.1-Channel HD Audio CODEC, Intel Dual Gigabit LAN port, 7 x PCIe 3.0/2.0 x16 slots |  |
| CPU | 10 cores, 20 threads, 3.3GHz (up to 4.3GHz) Intel Core i9-7900X 3.30GHz 13.75MB Cache 10C/20T Processor up to 4.30GHz turbo + Intel Liquid CPU Cooler Solution |  |
| RAM | 128GB - Kingston 128GB DDR4-2666MHz RAM (8 x 16GB – QVL memory) |  |
| SSD | 0.5TB - Samsung 970 Pro 512GB M.2 NVMe SSD 1TB - Samsung 970 Evo Plus 1TB M.2 NVMe SSD | Don’t store bulky data in home directory due to limited capacity. 1TB mounted to /data4 Empty Slots: - 7 SATA ports (additional disk enclosure needed for 5th~8th disks) |
| HDD | 4TB - 3.5” Seagate IronWolf 4TB 64MB Cache SATA Hard Disk Drive 8TB - 3.5” Seagate IronWolf 8TB 256MB Cache SATA Hard Disk Drive | Mounted to /data1 and /data2 Mounted to /data3 Store cold data on this HDD. |
| PSU | Enermax Fully Modular 1700W ATX 80+ Platinum Power Supply |  |
| Case | - Corsair Obsidian Series 750D Airflow Full Tower Chassis - Internal DVDRW SATA drive |  |
| GPU | Installed: - 2 X Zotac nVidia RTX2080Ti 11GB GDDR6 PCIe x16 GPU Card (Blower Edition) - 1 X Zotac nVidia RTX2080Ti 11GB (borrowed from Toh) Removed - 2 X Zotac nVidia RTX2080Ti 11GB GDDR6 PCIe x16 GPU Card (Blower Edition) potentially problematic, taken out for further check (No.1 not working, No.4 overheat.) |  |
| Cost (S$, inc GST) |  | 11,637.32 |

- Tower Server 3 (deployed in CNCL, coordinator: Yuting <yuting.wu@ntu.edu.sg>)

| Component | Description | Usage notes |
| --- | --- | --- |
| Domain name | server3.ntuiot.xyz |  |
| Motherboard | Gigabyte TRX40 AORUS XTREME Motherboard with integrated AMD TRX40 Chipset, integrated 2 x Intel 10Gigabit LAN Port, integrated Realtek Al1220-VB 7.1-Channel High Definition Audio CODEC |  |
| CPU | 64 cores, 128 threads, 2.9GHz (up to 4.3GHz) AMD Ryzen ThreadRipper 3990X CPU |  |
| RAM | 128GB - 128GB(8x16GB)DDR4-3 2 0 0 MHz |  |
| SSD | 1TB - M.2 NVMe SSD |  |
| HDD | 4TB - 1 x 3.5” 4TB Enterprise SATA Hard Disk Drive 8TB - 3.5” Seagate IronWolf 8TB 256MB Cache SATA Hard Disk Drive | /data1 2T /data2 2T /data3 8T |
| PSU | Fully Modular 2000W 80+ Approved Power Supply |  |
| Case | - Full Tower Chassis with Dual Fans |  |
| GPU | - GPU0: GV-N108TAORUS X-11GD 1 AORUS GeForce® GTX 1080 Ti Xtreme Edition 11G - GPU1: Quadro P5000 16GB (donated by NVIDIA) (temporally taken out) |  |
| Cost (S$, inc GST) |  | 21,211.68 |

- Tower Server 4 (deployed in HP-NTU Corp Lab, coordinator: Yimin Dai)

| Component | Description | Usage notes |
| --- | --- | --- |
| Domain name | server4.ntuiot.xyz |  |
| Motherboard | Asus WS C621E SAGE Motherboard with integrated Intel C621 Chipset, integrated 2 x Intel Gigabit LAN Port, integrated Realtek S1220A 7.1-Channel High Definition Audio CODEC |  |
| CPU | 24 cores, 48 threads, 3.3GHz (up to 4.2GHz) - CPU0: Intel® Xeon® Gold 6246 Processor, 24.75M Cache, 3.3 GHz - CPU1: Intel® Xeon® Gold 6246 Processor, 24.75M Cache, 3.3 GHz |  |
| RAM | 768GB - 768GB(12x64GB)DDR4-29 3 3 MHz Reg. ECC Memory |  |
| SSD | 1TB - 1 x Samsung 970 Pro 1TB M.2 NVMe SSD | /home |
| HDD | 4TB - 1 x 3.5” Seagate IronWolf 4TB 64MB Cache SATA Hard Disk Drive | /data1 2T /data2 2T |
| PSU | Fully Modular 2000W 80+ Approved Power Supply |  |
| Case | - Full Tower Chassis with Dual Fans |  |
| GPU | 4 X PNY nVidia Quadro RTX 8000 48GB GDDR6 PCIe x16 GPU Card(One GPU is broken and taken out) |  |
| Cost (S$, inc GST) |  | 48,952.50 |

- Tower Server 5 (deployed in Innovation Lab, coordinator: Zimo Ma)

| Component | Description | Usage notes |
| --- | --- | --- |
| Domain name | server5.ntuiot.xyz |  |
| Motherboard | Asus X299 SAGE |  |
| CPU | 10 cores, 20 threads, 3.3GHz - Intel® i9-9820X Processor, 16.5M Cache, 3.3 GHz |  |
| RAM | 768GB - 64GB(4x16GB) DDR4-2666 – QVL memory |  |
| SSD | 2TB - 1 x Samsung 1.92TB PCI-e - 1 x SX8200ProM.2 | / 1.1T /data 2T |
| HDD |  |  |
| PSU |  |  |
| Case |  |  |
| GPU | 4 X RTX2080Ti 11GB GDDR6 PCI-e x16 GPU |  |
| Cost (S$, inc GST) |  |  |

- 3 x Dell R730 server (pending moving, not ready for use yet)

- Two servers failed. Cannot boot up. Seems that they have memory problems.

- We have left all three servers on to check whether they can come back to live after warming up. (updated on 24 Nov 2020).

Following are not shared resources

- 6 x HP Z6 G4 Workstations in HP-NTU Corp Lab (in delivery)

| Component | Description | Usage notes |
| --- | --- | --- |
| Domain name |  |  |
| Motherboard |  |  |
| CPU | Intel Xeon 4214 2.2 2400MHz 12C 85 1st CPU |  |
| RAM | 32GB - Intel® C622 Chipset - 32GB (2x16GB) DDR4 2933 DIMM ECC Registered 1CPU Memory |  |
| SSD | 1.25TB - HP Z Turbo Drive M.2 256GB TLC SSD (loaded w OS) - HP Z Turbo Drive M.2 1TB TLC 2nd SSD |  |
| HDD |  |  |
| PSU | 1000W |  |
| Case | HP Z6 G4 90 1000W APJ Chassis |  |
| GPU | 1 x NVIDIA Quadro RTX 4000 8GB 3DP+USBc GFX 1st <3 DisplayPort outputs> |  |
| Cost (S$, inc GST) |  | 50,300.70 |